Continuous Assessment Test – II

Programme Name & Branch: B.Tech CSE

| Course Name & Code: CSE1003 & Digital Logic and Design |  |  |
| --- | --- | --- |
| Exam Duration: 90 Minutes Slot: C1+TC1 |  | Maximum Marks : 50 |

| Answer All Questions PART A 4 x 5 = 20 Marks |  |  |
| --- | --- | --- |
| Q.No | Question | Marks |
|  | Consider the following combinational circuit, | 5 (CO3) |
|  | Design an octal to binary encoder with its logic circuit, truth table and its output Boolean function. | 5 (CO4) |
|  | Convert SR flip-flop into JK flip-flop using truth table and excitation table. Draw the converted JK flip-flop with necessary inputs. | 5 (CO5) |
|  | Calculate and Compare the Total Propagation Time of 4-bit Ripple Carry Binary Parallel Adder (BPA) and 4-bit BPA with Look-Ahead Carry Generator for the following timing specifications: a. XOR - 40ns b. AND – 25ns c. OR – 15ns | 5 (CO4) |
| Answer All Questions PART – B 3 x 10 = 30 Marks |  |  |
| Q.No | Question | Marks |
|  | An ABCD-to-seven-segment decoder is a combinational circuit that converts a decimal digit in BCD to an appropriate code for the selection of segments in an indicator used to display the decimal digit in a familiar form. The seven outputs of the decoder (a, b, c, d, e, f, g) select the corresponding segments in the display, as shown in Figure (a) . The numeric display chosen to represent the decimal digit is shown in Figure (b) . Using a truth table and Karnaugh maps, design the BCD-to-seven-segment decoder using a minimum number of gates. The six invalid combinations in BCD should result in a blank display. Figure (a) Figure (b) | 10 (CO3) |
|  | Implement the following Boolean function using multiplexer - F1A,B,C,D= ∑(0,2,5,8,10,14) - F2A,B,C,D= ∑(0, 1, 2, 4, 5, 6, 8, 9, 10, 11, 13, 14, 15) | 10 (CO4) |
|  | Design a full adder using 3 to 8 line decoder. The decoder uses only the basic AND and NOT gates. - Draw the logic circuit of full adder - Give the truth table for full adder output Sum and Carry - Derive the Boolean expression for D0 through D7, Sum and carry | 10 9CO4) |