dIgital logic and design

LAB ASESSMENT – 5

Name: VIBHU KUMAR SINGH

Reg. No: 19BCE0215

Teacher: Sairabanu J.

Q1) Realization of characteristic table of different types of flip flop.

A1)

- S-R Flip Flop

Logic Diagram:

Characteristic Table:

| Q | S | R | Q(t+1) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | Forbidden(0) |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | Forbidden(0) |

S-R Flip Flop:

Logging Table:

- D Flip Flop

Logic Diagram:

Characteristic Table:

| Q | D | Q(t+1) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

D Flip Flop:

Logging Table:

- JK Flip Flop

Logic Diagram:

Characteristic Table:

| Q | J | K | Q(t+1) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 0 |

JK Flip Flop:

Logging Table:

- T Flip Flop

Logic Diagram:

Characteristic Table:

| Q | T | Q(t+1) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

T Flip Flop:

Logging Table:

Q2) Design and implement a 3 bit binary synchronous counter using T flip Flop.

A2)

Excitation Table:

| Present State |  |  | Next State |  |  | Flip-Flop |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A2 | A1 | A0 | A2 | A1 | A0 | TA2 | TA1 | TA0 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |

Equations:

TA2=A1A0

TA1=A0

TA0=1

Circuit:

Logging Table:

Q3) Design a SISO Shift register.

A3)

Truth Table:

| Clock pulse No | QA | QB | QC | QD |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 2 | 0 | 1 | 0 | 0 |
| 3 | 0 | 0 | 1 | 0 |
| 4 | 0 | 0 | 0 | 1 |
| 5 | 0 | 0 | 0 | 0 |

Screenshots(4):

Logging Table:

Q4) Design the state diagram using JK flip flops.

A4)

Equations:

JA=BX’

KA=BX

JB=X

KB=A’X’+AX=A (XNOR) X

K-Maps:

Screenshots:

Logging Table: